Assembler Symbols

-----------------

The assembler symbols are stored at the end of the data buffer, eg while

the assembled binary grows to higher memory the symbols grow to lower

memory. Free memory is the area in between.

The symbols are hashed based on the low byte of the CRC for the label. The

symbols for individual hashes are then separate chained.

In practice the following variables are kept in the assembler variable

buffer, which is of fixed size:

Name		Nibbles		Description

----		-------		-----------

O_LABELS	1		<>0 if any symbols exist

O_DEFINES	1		<>0 if any of the symbols is a DEFINE

O_MACROS	1		<>0 if any of the symbols is a MACRO

O_NEXTNODE	5		Address of the newest symbol

O_HASHEND	5		Address of the end of the data buffer

O_HASHES	256*5		Addresses of the latest symbols

for individual hashes.

Each of the symbol definitions looks like this:

REL(5)	nextsymbol		Offset to the next symbol with the

same hash

CON(1)	symblen			Name length, 0-15 for 1-16 chars

NIBASC	'name'			Name itself

CON(1)	type			Type of symbol

<type specific data>

When a new symbol is added, following is done:

1) Hash is calculated for the name.

2) Calculate required size of the new defintion

3) The address for the latest symbol by that hash is fetched

from O_HASHES.

4) If the address is null then

Set the address to be NEXTNODE - size

Set NEXTNODE to be NEXTNODE - size

Else

Traverse all the symbols for that hash, looking for

a matching name. If none is found then

Set node address to be that match

else

Set node address to be NEXTNODE - size

Set NEXTNODE to be NEXTNODE - size

5) If we created a new node then link the new node to the previous

one indicated in O_HASHES. If there was none then the link is

zero.

6) If an old symbol was found by the same name then

Depending on old type and new type either error or

redefine old value

Else

Write symbol lenght, name, type and type specific data

For example consider that symbols NAME1 and NAME2 have the same hash. Then

after their addition the symbols area will look like this:

Node1	REL(5)	Node2		Link to Node2

CON(1)	5-1		5 chars in name

NIBASC	'NAME1'

<type and type specific data>

Node2	CON(5)	0		No more symbols with the same hash

CON(1)	5-1		5 chars in name

NIBASC	'NAME2'

<type and type specific data>

NodesEnd

- O_NEXTNODE will point to Node1 because it was the latest added node.

- One of the addresses in O_HASHES will point to Node1, because it was the

latest added node for that hash.

- O_HASHEND will point to NodesEnd, since it is the end of buffer.

When a symbol is fetched, following is done:

1) Hash is calculated for the name

2) The address for the latest symbol by that hash is fetched

from O_HASHES.

3) If address is zero then no such symbol exist

4) Else the chained symbols are traversed to find a symbol with

the same name, search terminates if a zero link is found.

5) If no match was found then symbol is not defined.

Else the address of the type field is returned.

The type specific data for various nodes is:

Labels

------

- Cannot be redefined

- Value is relative

- Allowed in expressions

CON(1)	SYMB_LABEL		Type

CON(1)	5-1			5 nibbles

CON(5)	address			Offset from start of output binary

Equates

-------

- Cannot be redefined

- Value is absolute

- Allowed in expressions

CON(1)	SYMB_EQU		Type

CON(1)	16-1			16 nibbles

CON(16)	value			Value of equate

Redefinable equates

-------------------

- Can be redefined

- Value is absolute

- Allowed in expressions

CON(1)	SYMB_=			Type

CON(1)	16-1			16 nibbles

CON(16)	value			Value of equate

Propsymbols

-----------

- Cannot be redefined

- Value is absolute

- Allowed in expressions

- Purpose is to provide access to the command numbers for commands

CON(1)	SYMB_PROP		Type

CON(1)	6-1			6 nibbles

CON(6)	#1000*(cmdnum)+(romid)	Value for prop

Hash symbols

------------

- Provides means to define secondary hashes for commands

CON(1)	SYMB_HASH		Type

CON(3)	cmdnum			Associated command number

CON(1)	chars-1			Length of hash

NIBASC	'hash'			Hash

Romp symbols

------------

- Provides means to define romptr names, locations and hashes

CON(1)	SYMB_ROMP		Type

CON(5)	address			Address of romp relative to DOLIB

CON(3)	cmdnum			Associated command number

CON(1)	visible?		Flag to indicate NULLNAME/other

CON(1)	chars-1			Length of hash

NIBASC	'hash'			Hashname, 16 char slot

Code symbols

------------

- Provides means to fix code object length fields.

When CODE is encountered in pass 2 the code label defined by the

current output address is fetched and to get the address for

then end of the code object (defined in pass 1)

CON(1)	SYMB_CODELAB		Type

CON(1)	5-1			5 nibbles

CON(5)	address			Address of endcode relative to output

Define symbols

--------------

- Provides means to define symbols to have source code values

CON(1)	SYMB_DEFINE		Type

NIBHEX	string			The defined value

CON(5)	=DOCSTR			This is only for GC

REL(5)	endofbuf		Link to end of buffer

Macro symbols

-------------

- Provides means to define machine language macros

CON(1)	SYMB_MACRO		Type

CON(1)	flag			Flag to indicate expansion

NIBHEX	string			The macro value

CON(5)	=DOCSTR			This is only for GC

REL(5)	endofbuf		Link to end of buffer